State-of-the-Art Report

Energy-Efficient Secure Clustering in WSNs Using Hybrid Cryptography — Complete Results, Sources, and Improvement Rationale

This report compiles every numeric result (FND/HND/LND/PDR) produced or cited in this thesis — from each protocol's original paper, through this thesis's own paper-faithful (NS-3 Original) and unified-environment (NS-3 Edited) reproductions, to every improvement this thesis made and WHY each improvement worked. It closes with the thesis's own Proposed Protocol (V3, V5 and the final optimized v8).

# Research Journey Overview

Wireless sensor network clustering protocols make very different trade-offs between energy efficiency, network lifetime, and delivery reliability, and published results for the same protocol often come from different simulators, field sizes, and radio models, which makes head-to-head comparison unreliable if taken at face value. This report's journey therefore begins by reproducing three foundational clustering protocols — LEACH, HEED, and PEGASIS — inside a single, consistent NS-3 environment, so that their FND, HND, LND and PDR numbers could be measured against each other under identical conditions rather than compared across incompatible papers.

With that common baseline established, the work turned to two hybrid protocols already proposed in the literature, SH-LEACH and H-LEACH, both of which combine LEACH-style clustering with HEED-style cost-based CH selection. Reproducing each one faithfully first, then checking its behaviour against the improvements its own paper claimed, uncovered genuine gaps between what each algorithm was described as doing and what its pseudocode could mathematically achieve — H-LEACH's strict energy threshold could never be satisfied under equal starting energy, and SH-LEACH's CH-selection formula applied only a single doubling instead of the iterative doubling its description implied. Correcting each gap produced the “Edited” versions of both protocols, which is why every literature-hybrid table in this report carries three rows rather than one.

Having diagnosed both what the classic baselines could and could not do, and where the literature's own hybrids fell short, the thesis then developed an original protocol family of its own, evolved through explicit, numbered versions (v1 through v7.2, then the optimized v8). Each version changes exactly one mechanism — CH-selection formula, cluster-reuse interval, backup failover, relay chaining, or multihop routing — so that its individual effect on FND/HND/LND/PDR could be isolated and attributed rather than lost in a bundle of simultaneous changes. The overall arc of the journey, detailed step by step in the sections that follow, is that architectural changes to how a cluster head is chosen and how it communicates mattered far more than any single parameter tweak, and that no version optimizes every metric at once — the final configuration is therefore a choice between a best-balanced option and a best-reliability option, not a single “winner.”

# 1. LEACH (2000)

Heinzelman, Chandrakasan & Balakrishnan — Energy-Efficient Communication Protocol for Wireless Microsensor Networks — HICSS 2000

• FND: 932 rounds (at 0.5 J, Table 2)

• LND: 1312 rounds

• Improvement vs. direct transmission: up to 8× lower energy consumption

| Source | FND | HND | LND | PDR |
| --- | --- | --- | --- | --- |
| Original Paper | 932 | N/A | 1312 | N/A |
| Our NS-3 Original (paper-exact: 50×50m, far BS, single-term radio) | 882 | 970 | 1321 | 98.02% |
| Our NS-3 Edited (unified: 100×100m, BS center, two-slope radio) | 1409 | 1626 | 1962 | 98.37% |

Note: The Edited (unified) numbers differ from the paper-exact numbers ONLY because the environment changed (field size, BS position, radio model) — the algorithm itself is identical in both rows.

# 2. HEED (2004)

Younis & Fahmy — HEED: A Hybrid, Energy-Efficient, Distributed Clustering Approach — IEEE TMC 2004

• Results: Graph only (Fig. 8a/8b) — no exact numeric table

• Qualitative finding: HEED consistently outperforms gen-LEACH across 300–700 nodes, no crossover

| Source | FND | HND | LND | PDR |
| --- | --- | --- | --- | --- |
| Original Paper | Graph only | N/A | Graph only | N/A |
| Our NS-3 Original (paper-exact: Table 2, far BS, E0=2J, packet=800bit) | 1106 | 1626 | 1996 | 99.97% |
| Our NS-3 Edited — Original variant (unified env.) | 323 | 570 | 1362 | 99.69% |
| Our NS-3 Edited — Modified variant (unified env. + rotation-fairness penalty) | 338 | 549 | 1502 | 99.67% |

WHY this improvement happened: The Modified variant adds a fairness penalty that discourages the same node from repeatedly winning CH selection. Spreading the CH role more evenly across nodes delays the point at which any single overworked node dies, raising LND from 1362 to 1502 (+10.3%) with no cost elsewhere.

Note: The paper itself provides no exact FND/HND/LND table; the two Edited rows are both this thesis's own designs.

# 3. PEGASIS (2002)

Lindsey & Raghavendra — PEGASIS: Power-Efficient Gathering in Sensor Information Systems — IEEE Aerospace Conference 2002

• FND (50×50m, 0.5J): 1578 rounds (Table 1)

• HND: 2082 rounds

• LND: 2192 rounds

| Source | FND | HND | LND | PDR |
| --- | --- | --- | --- | --- |
| Original Paper | 1578 | 2082 | 2192 | N/A |
| Our NS-3 Original (paper-exact: 50×50m, far BS, single-term radio) | 1614 | 2036 | 2167 | 97.75% |
| Our NS-3 Edited (unified: 100×100m, BS center, two-slope radio) | 1324 | 2353 | 2887 | 98.67% |

Note: The closest numeric match to any original paper reproduced in this thesis (under 2.3% difference across all three metrics, paper-exact row).

# 4. SH-LEACH (2015) — LEACH+HEED Hybrid

Shrestha, Kim, Jung & Lee — The Improved Energy Efficient LEACH Protocol Technology of WSNs — IJIBC 2015

• Results: Graph only (Figure 2) — no exact numeric table

• Qualitative finding: SH-LEACH's curve sits above LEACH's in early rounds (delayed FND); converges with LEACH by late rounds (similar LND)

| Source | FND | HND | LND | PDR |
| --- | --- | --- | --- | --- |
| Original Paper | Graph only | N/A | Graph only | N/A |
| Our NS-3 Original (paper-exact) | 493 | 747 | 884 | 99.53% |
| Our NS-3 Edited (unified env., before improvement) | 1440 | 1584 | 1604 | 99.17% |
| Our NS-3 Edited + Improvement (true iterative doubling, Cprob 0.10→0.03) | 1247 | 1600 | 1649 | 99.79% |

WHY this improvement happened: This thesis found the CH-selection formula only applied a SINGLE fixed doubling of the base probability rather than the TRUE iterative HEED-style doubling the paper's own description implies. Switching to genuine iterative doubling, and re-tuning Cprob down (fewer, larger clusters reduce the total number of costly long-range transmissions), improved HND/LND/PDR modestly — though FND dropped 13.4%, a genuine trade-off: fewer clusters means each surviving CH carries a heavier aggregation load, so the very first CH tends to die a bit sooner even as the network as a whole lasts longer.

# 5. H-LEACH (2016) — LEACH+HEED Hybrid

Razaque, Mudigulam, Gavini, Amsaad, Abdulgader & Krishna — H-LEACH: Hybrid-Low Energy Adaptive Clustering Hierarchy for WSNs — IEEE LISAT 2016

• LEACH's result in the same experiment: last node died at ≈1778 rounds (read from Fig. 2 graph)

• H-LEACH's result: last node died at ≈4312 rounds (read from Fig. 2 graph)

| Source | FND | HND | LND | PDR |
| --- | --- | --- | --- | --- |
| Original Paper (approx. from graph) | N/A | N/A | ~4312 | N/A |
| Our NS-3 Original (paper-exact, WITH required fallback) | 1080 | 1160 | 1210 | 94.75% |
| Our NS-3 Edited (unified env., before improvement) | 2177 | 2255 | 2305 | 97.33% |
| Our NS-3 Edited + Improvement (energy-gate relaxed 1.0×→0.8× of average) | 2347 | 2449 | 2499 | 97.33% |

WHY this improvement happened: The paper's own algorithm, as literally written, never elects a single cluster head: all nodes start with EXACTLY equal energy, so the network average equals every node's own energy, and the paper's strict "energy > average" condition can never be satisfied by a strict inequality — a permanent mathematical deadlock. A necessary fallback (promote the highest-energy node) was added just to get the algorithm running at all. Even with that fallback, the fallback itself fired far more often than intended (the paper's own "smart" energy-weighted selection was rarely actually deciding anything). Relaxing the gate to 0.8× let the intended energy-weighted mechanism govern selection more often, improving FND/HND/LND by 7.8-8.6% with zero cost to PDR.

# 6. EECH-HEED (2025) — HEED+EECH Hybrid (not LEACH+HEED)

Kaur, Kour & Singh — EECH-HEED: An Adaptive Hybrid Clustering Protocol for Energy Efficient Soil Monitoring — Scientific Reports 2025

• FND: 1250 rounds (Table 7)

• HND: 1650 rounds

• LND: 2200 rounds

• PDR: 95%

| Source | FND | HND | LND | PDR |
| --- | --- | --- | --- | --- |
| Original Paper | 1250 | 1650 | 2200 | 95% |
| Our NS-3 Original (paper-exact, packet=4000bit) | 792 | 1079 | 1129 | 90.43% |
| Our NS-3 Edited (unified env., packet=2000bit, before improvement) | 1653 | 2134 | 2184 | 91.68% |
| Our NS-3 Edited + Improvement (added Adaptive Threshold-Based Sensing) | 2502 | 3758 | 3808 | 92.14% |

WHY this improvement happened: The paper's own design includes an Adaptive Threshold-Based Sensing mechanism (nodes only transmit when a sensed reading changes meaningfully, or after a maximum silent interval) — but this thesis's initial reproduction never implemented it, so every node transmitted every round unconditionally. That single missing mechanism is why the initial reproduction (1129) fell below the paper's own number (2200). Implementing it (with thresholds re-tuned specifically for this file's 2000-bit packet size, after an earlier attempt using thresholds tuned for the paper's 4000-bit packet caused zero deaths at all in 5000 rounds) produced the highest raw lifetime of any protocol in this entire thesis.

Note: This protocol hybridizes HEED with EECH, not with LEACH — used here as broader comparative context, not as a direct peer of the Proposed Protocol.

# 7. Broader Related Work (Context Only, Not Reproduced)

These recent hybrid protocols were reviewed for context but not reproduced in NS-3, as none combines LEACH with HEED specifically.

# 7.1 TLC-LEACH (2024)

Subedi, Acharya, Lee & Lee — Two-Level Clustering Algorithm for Cluster Head Selection in Randomly Deployed WSNs — Telecom (MDPI) 2024

• Dead-node reduction at round 200: 9.72% better than I-LEACH, 16.52% better than LEACH-E, 195.95% better than LEACH

• Throughput at round 5000: 10% higher than a competing heterogeneous routing method

Note: Addresses LEACH's "boundary problem" via an angular-distance criterion.

# 7.2 DL-HEED (2025)

Juwaied & Jackowska-Strumillo — DL-HEED: A Deep Learning Approach to Energy-Efficient Clustering in Heterogeneous WSNs — Applied Sciences (MDPI) 2025

• LND improvement: 21.7% (50 nodes) to 44.4% (500 nodes)

• PDR improvement: 93.2%→97.5% (50 nodes), 82.5%→91.3% (500 nodes)

Note: Integrates a Graph Neural Network (GNN) into HEED's CH selection.

# 7.3 RL-ILEACH (2026)

El-Sayed, Abd-Elgaber & Refaay — An Intelligent RL Enhanced Improved LEACH Protocol for Prolonging WSN Lifetime — Scientific Reports 2026

• Lifetime improvement at 500 nodes: +784% vs. LEACH, +130% vs. ILEACH

• PDR result: not statistically significant (p=0.140/0.550) despite significant energy and throughput gains (p<0.001)

Note: Integrates lightweight tabular Q-learning into ILEACH.

# Comparative Results Discussion

Reading each protocol's tables in isolation makes the individual numbers clear, but the connected story — how one generation of results led to the next — is easiest to see when the three families are walked through in the order they were actually studied: reference baselines, then literature hybrids, then the thesis's own proposed evolution.

The three reference protocols set very different ceilings. LEACH, the original randomized-rotation clustering protocol, reached FND/HND/LND of 1409/1626/1962 rounds with a PDR of 98.37% under this thesis's unified environment. HEED, known for replacing LEACH's pure randomness with a hybrid energy-and-communication-cost CH selection that avoids expensive long-range re-election broadcasts, achieved a far lower raw lifetime (FND 323, HND 570, LND 1362) but the family's highest reliability at 99.69% PDR — a direct consequence of its more disciplined, cost-aware CH negotiation. PEGASIS, known for chaining all nodes into a single data-gathering path instead of forming clusters at all, produced the strongest raw lifetime of the three (FND 1324, HND 2353, LND 2887, PDR 98.67%), at the cost of the latency and single-point-of-failure risk inherent to a chain topology.

Building on that baseline picture, the two literature hybrids show what happens when LEACH's clustering is combined with a HEED-style cost formula. H-LEACH (Edited) reached FND/HND/LND of 2177/2255/2305 with PDR 97.33% — relaxing its energy-selection gate to 0.8× let the intended energy-weighted mechanism actually govern CH selection instead of falling back to a default rule almost every round, lifting every lifetime metric by 7.8–8.6% for no PDR cost. SH-LEACH (Edited) reached 1440/1584/1604 rounds with PDR 99.17% — switching from a single fixed probability doubling to genuine iterative HEED-style doubling, and re-tuning cluster probability downward, modestly improved HND/LND/PDR, though FND fell 13.4% because fewer, larger clusters mean each surviving CH carries more aggregation load and tends to die slightly sooner.

The thesis's own protocol family then carried this lesson further, isolating one mechanism per version across eleven steps. The single largest jump in the entire journey came at v2→v3, where HEED's iterative negotiation was replaced by a one-pass energy×connectivity score recomputed only once every five rounds — a genuine architectural fix that raised LND by 94% rather than a parameter tweak. That gain proved fragile to careless tuning: v3→v4 simply stretched the reuse interval to 15 rounds and every metric regressed, because a longer interval means a longer unprotected outage whenever a cluster head dies. v4→v5 recovered most of that loss for free by pre-designating a backup CH with zero extra messaging cost. From v5 onward the family explored network-level fixes rather than selection-formula fixes: a CH-to-CH relay chain (v6) raised PDR at both a central and a far base-station placement and gave the far-BS configuration its best FND by shortening each CH's average hop distance; adding backup failover and chain repair (v7) let a promoted backup inherit its dead predecessor's position and repair the relay path rather than break it, lifting PDR further at both placements; and replacing the single ordered chain with a greedy multihop routing tree (v7.1) removed the last single-gateway bottleneck to reach the family's highest PDR of 98.49%, before v7.2 confirmed, by lengthening the same multihop architecture's setup interval, that reuse-interval length is a lever independent of topology or CH-selection quality.

| Family | Representative Version | FND | HND | LND | PDR | Note |
| --- | --- | --- | --- | --- | --- | --- |
| Baseline Protocols | PEGASIS | 1324 | 2353 | 2887 | 98.67% | Highest raw lifetime among the three baselines; HEED instead achieves the baseline family's highest PDR (99.69%) at a much shorter lifetime. |
| Literature Hybrids | H-LEACH (Edited) | 2177 | 2255 | 2305 | 97.33% | Highest lifetime among literature hybrids; SH-LEACH (Edited) instead achieves the hybrid family's highest PDR (99.17%). |
| Proposed Protocol (Balanced) | v7 (Center) | 1551 | 1976 | 2066 | 98.432% | Best-balanced configuration across all four metrics. |
| Proposed Protocol (Peak Reliability) | v7.1 (Multihop, Int=5) | 1491 | 1991 | 2086 | 98.49% | Highest PDR of the entire proposed family, at a small FND cost. |
| Proposed Protocol (Optimized) | v8 (Center) | 2621 | 2711 | 2761 | 99.22% | Best FND/HND/PDR of every protocol tested at the center BS; +33% delivered packets vs v5b for the same 50 J. |

# 8. This Thesis's Proposed Protocol — Full Evolution (V1-V8)

Original contribution of this thesis — not from external literature

Root-cause diagnosis: HEED's iterative CH-negotiation (Section 2) is charged in full every single round, regardless of whether the network's topology actually changed — this overhead, not HEED's selection quality, is why HEED loses to plain LEACH in raw lifetime. The table below shows the full confirmed evolution from the first hybrid attempt (V1) through the combined designs (V7, V7.1/V7.2) and the final optimized design (V8), all via real NS-3 runs.

| Version | Idea | FND | HND | LND | PDR |
| --- | --- | --- | --- | --- | --- |
| v1 | HEED-style CH selection + LEACH clustering, no multi-round reuse | 330 | 557 | 1318 | 99.74% |
| v2 | + Rotation fairness penalty | 345 | 534 | 1393 | 99.79% |
| v3 | Single-pass Energy × Connectivity score + Multi-round reuse | 1730 | 2081 | 2703 | 97.72% |
| v4 | Longer setup interval: 5 → 15 rounds | 1513 | 2219 | 2521 | 94.67% |
| v5 | + Backup CH Failover | 1513 | 2127 | 2416 | 97.05% |
| v5-exp (Interval=5) | v3's interval (5) + Backup CH Failover + Chain/Cluster Repair | 1730 | 2086 | 2156 | 98.27% |
| v5-exp (Interval=10) | Interval=10 + Backup CH Failover + Chain/Cluster Repair | 1621 | 2141 | 2291 | 97.94% |
| v6 (Center) | CH-to-CH Chain; BS at field center | 1551 | 1981 | 2106 | 97.494% |
| v7 (Center) | Chain + Backup Failover + Chain Repair | 1551 | 1976 | 2066 | 98.432% |
| v6 (Far BS) | CH-to-CH Chain; Far BS | 1266 | 1910 | 2031 | 97.075% |
| v7 (Far BS) | Chain + Backup Failover + Chain Repair; Far BS | 1266 | 1891 | 1976 | 98.135% |
| v7.1 (Multi-hop Int5) | Multi-hop routing + Backup Failover + Routing Repair (Setup Interval = 5 rounds) | 1491 | 1991 | 2086 | 98.49% |
| v7.2 (Multi-hop Int10) | Multi-hop routing + Backup Failover + Routing Repair (Setup Interval = 10 rounds) | 1381 | 2071 | 2241 | 97.878% |
| v5b (Interval=5) | v5-exp (Interval=5) + Energy-Aware Repair (members/backups below 5% of E0 are not reconnected/promoted) | 1730 | 2091 | 3203 | 97.31% |
| v8 (Center) | v5b + epoch-exhaustion fix + proactive CH handover + orphan re-join + direct-to-BS + energy-gated CH election + relative low-energy threshold | 2621 | 2711 | 2761 | 99.22% |
| v8 (Far BS) | Same v8 code; BS far (50,-100) | 1471 | 1646 | 1716 | 98.66% |

Note: v6 has two variants in different environments (Far-BS / Center) — a controlled experiment isolating the Sink's location from the chain mechanism itself, not two sequential versions. v7 must be compared against v6 Far-BS specifically (same environment), not v3.

WHY these results happened: v1→v2: a rotation-fairness penalty on HEED's cost formula gave a small, consistent gain (LND +5.7%). v2→v3: replacing HEED's iterative negotiation with a single-pass energy×connectivity score, plus recomputing CHs only once every 5 rounds, produced the decisive jump (LND +94%) — a genuine architectural fix, not a parameter tweak. v3→v4: stretching the interval to 15 rounds alone regressed every metric (LND -6.7%, PDR -3pp) — longer intervals without protection mean longer outages when a CH dies. v4→v5: a pre-designated backup CH (zero extra messaging cost) recovered most of v4's reliability loss (PDR 94.67%→97.05%). v5→v6: introducing a CH-to-CH relay chain, tested under IDENTICAL field/radio conditions at both BS placements, raised PDR at Center (97.05%→97.49%) and gave the family's Far-BS configuration its FND of 1266 by shortening each CH's average hop distance. v6→v7: adding backup failover + chain repair on top of the chain lets a promoted backup inherit its dead predecessor's exact position in the chain, repairing the relay path instead of breaking it — raising PDR further in BOTH placements (Center 97.49%→98.43%, Far-BS 97.08%→98.14%) at a small LND cost (-1.9% center, -2.7% far). v7→v7.1 (Multihop, Int=5): replacing the single ordered chain with a greedy multihop routing tree removes the single-gateway bottleneck and achieves the highest PDR of the entire family (98.49%), though FND falls slightly (1551→1491) because more CHs now share intermediate relay duty. v7.1→v7.2 (Multihop, Int=10): lengthening the setup interval within the SAME multihop architecture trades some reliability for lifetime (PDR 98.49%→97.88%, LND 2086→2241) — confirming, as in v3→v4, that setup frequency is a separate lever from CH-selection or relay-topology quality. v5-exp → v5b → v8: v5b's LND of 3203 is misleading — its last 1038 rounds delivered only 59 packets, because Energy-Aware Repair left weak nodes alive but silent and a latent epoch-exhaustion bug left 998 rounds with zero CHs. v8 fixes the bug, protects CHs BEFORE they die (proactive handover) as well as after (backup failover), and removes load from CHs (direct-to-BS, energy-gated election): FND 1730→2621 (+51%), HND 2091→2711 (+30%), PDR 97.31%→99.22%, and 33% more packets delivered with the same 50 J. Nodes now die close together (FND–LND spread 140 rounds), which is why LND is lower than v5b's inflated value.

8.1 Controlled Variant Experiments (Chain vs. Backup vs. No-Chain vs. Multihop, Center vs. Far-BS)

This session rebuilt v6 and v7 so Center and Far-BS share an IDENTICAL field/radio model (100x100m, two-slope radio, RANGE=25) -- only the BS's (x,y) position differs -- turning the earlier v6/v7 comparison (which mixed different environments) into a clean, controlled 2x2 experiment (mechanism x BS position). Two further structural variants were added: a no-chain, longer-interval version (direct CH->BS, backup only, interval=10) and a multihop routing-tree version (each CH forwards to the nearest OTHER CH strictly closer to the BS -- a forest of trees, no single gateway).

| Variant | Mechanism | Environment | FND | HND | LND | PDR | Interval |
| --- | --- | --- | --- | --- | --- | --- | --- |
| v6_center | Chain only, no backup | 100x100 unified, BS center (50,50) | 1551 | 1981 | 2106 | 97.49% | 5 |
| v6_farBS_v2 | Chain only, no backup | 100x100 unified, BS far (50,-100) | 1266 | 1910 | 2031 | 97.08% | 5 |
| v7_center | Chain + Backup + Chain Repair | 100x100 unified, BS center (50,50) | 1551 | 1976 | 2066 | 98.43% | 5 |
| v7_farBS_v2 | Chain + Backup + Chain Repair | 100x100 unified, BS far (50,-100) | 1266 | 1891 | 1976 | 98.14% | 5 |
| v7_center_noChain_int10 | No Chain (direct CH->BS) + Backup only | 100x100 unified, BS center (50,50) | 1621 | 2151 | 2321 | 97.78% | 10 |
| v7_center_multihop | Multihop routing tree (no single chain/gateway) + Backup | 100x100 unified, BS center (50,50) | 1491 | 1991 | 2086 | 98.49% | 5 |

WHY these results happened: with the SAME field/radio model, Far-BS variants show earlier FND/LND across the board than Center variants -- confirming BS placement, not the relay mechanism, drives the baseline lifetime gap. Within each BS placement, Backup+Chain-Repair (v7) consistently raises PDR over Chain-only (v6) by ~0.9-1.0 percentage points, at a negligible LND cost (-1.9% center, -2.7% far). The no-chain/interval=10 variant achieves the single highest LND (2321) of any controlled variant -- fewer setup rounds mean fewer vulnerable re-clustering transitions, confirming (as in v3-v4) that control overhead, not selection quality, is the dominant lever for raw lifetime. The multihop variant achieves the highest PDR of all variants tested (98.49%) -- removing the single-gateway bottleneck pays off even though the mechanism adds no extra energy budget for route discovery.

8.2 Robustness Bugfixes Applied This Session

• Bug found: in early v7 runs, ~10 rounds showed CH_Count=0 / Delivered=0 / PDR=0 for that round even while nodes were still alive -- the designated backup CH was rejected for insufficient energy to cover the WHOLE remaining setup interval, and in a rarer case the CH-selection fallback itself produced zero candidates when every alive node had already served as CH within the current epoch window.

• Fix A (CH-selection fallback): if the epoch-restricted fallback is ALSO empty, relax the restriction and pick from any alive node -- guarantees >=1 CH whenever >=1 node is alive.

• Fix B (two-tier graceful degradation): Tier 1 promotes the designated backup as soon as it can afford >=1 round (not the whole remaining interval), flagging a 'partial promotion' when it cannot cover the full interval; Tier 2 emergency-promotes the orphaned cluster's own highest-remaining-energy member if Tier 1 still fails. Result: zero CH_Count=0 anomalies across all re-run experiments; the no-chain/interval=10 variant genuinely triggered Tier 2 once (an 'emergency promotion').

8.3 v8 — Optimized Hybrid: Proactive + Reactive CH Protection

v8 keeps every v5b mechanism (single-pass energy × connectivity selection, 5-round reuse, pre-designated backup CH, chain/cluster repair, energy-aware repair) and adds five targeted mechanisms plus one threshold correction. Each can be disabled at compile time (-DI1=0 … -DI5=0) for the ablation below. All runs use the same unified environment (100 nodes, 100×100 m, E0=0.5 J, 2000-bit packets, two-slope radio, seed 12345).

• I1 — Epoch-exhaustion fix: when every alive node has already served as CH in the current epoch, a new epoch starts immediately. In v5b this situation produced 998 rounds with CH_Count=0 (e.g., rounds 196–200 with all 100 nodes alive and PDR=0). Same idea as Fix A in 8.2, now applied to the v5 branch.

• I2 — Proactive CH handover: before the data phase, a CH whose residual energy cannot cover this round's workload (members × (Rx+Agg) + Tx to BS) hands its role to its highest-energy member while still alive; one control packet is charged for the announcement. Backup failover (v5) still handles sudden deaths — prevention before, repair after.

• I3 — Orphan re-join: members whose CH died with no usable backup join the nearest surviving CH (already known from the setup advertisements, so no new messages).

• I4 — Direct-to-BS: a node that is no farther from the BS than from its CH (or that has no CH) transmits straight to the BS — its own TX cost is equal or lower, and the CH saves the RX + aggregation energy.

• I5 — Energy-gated CH election: nodes below the network's average residual energy may not volunteer as CH.

• Threshold correction — Energy-Aware Repair now compares against 5% of the live average energy instead of 5% of E0. With the fixed 0.025 J threshold every late-life backup was rejected (0 successful / 20 rejected); with the relative threshold 18 promotions succeed.

Table 8.3a — v8 vs. v5b and v7 under the same field/radio model (seed 12345):

| Variant | Mechanism | Environment | FND | HND | LND | PDR | Interval |
| --- | --- | --- | --- | --- | --- | --- | --- |
| v5b | Backup + Repair + Energy-Aware Repair | 100x100 unified, BS center (50,50) | 1730 | 2091 | 3203* | 97.31% | 5 |
| v7_center | Chain + Backup + Chain Repair | 100x100 unified, BS center (50,50) | 1551 | 1976 | 2066 | 98.43% | 5 |
| v8_center | v5b + I1–I5 + relative threshold | 100x100 unified, BS center (50,50) | 2621 | 2711 | 2761 | 99.22% | 5 |
| v5b_farBS | Backup + Repair + Energy-Aware Repair | 100x100 unified, BS far (50,-100) | 1055 | 1616 | 1861 | 98.28% | 5 |
| v7_farBS_v2 | Chain + Backup + Chain Repair | 100x100 unified, BS far (50,-100) | 1266 | 1891 | 1976 | 98.14% | 5 |
| v8_farBS | v5b + I1–I5 + relative threshold | 100x100 unified, BS far (50,-100) | 1471 | 1646 | 1716 | 98.66% | 5 |

* v5b LND is inflated by stranded nodes: its last 1038 rounds (≤10 nodes alive) delivered 59 packets in total. Total delivered packets over the whole lifetime: v5b 201,505 vs v8 268,610 (+33%) at the center BS; 157,675 vs 161,594 at the far BS. Across 8 random topologies (seeds 12345, 1, 7, 42, 99, 2024, 31337, 555) the mean center-BS results are v5b 1688/2119/2374/97.56% vs v8 2618/2720/2766/99.26%, and v8 had a higher LND than v5b in 7 of the 8 topologies.

Table 8.3b — Leave-one-out ablation of v8 (center BS, seed 12345): each row disables ONE mechanism.

| Variant | Mechanism removed | Delivered packets | FND | HND | LND | PDR | Direct-to-BS pkts |
| --- | --- | --- | --- | --- | --- | --- | --- |
| v8 (full) | — | 268,610 | 2621 | 2711 | 2761 | 99.22% | 118,986 |
| v8 − I1 | Epoch-exhaustion fix | 268,610 | 2621 | 2711 | 2761 | 99.22% | 118,986 |
| v8 − I2 | Proactive handover | 268,509 | 2585 | 2726 | 2766 | 98.98% | 118,748 |
| v8 − I3 | Orphan re-join | 268,610 | 2621 | 2711 | 2761 | 99.22% | 118,986 |
| v8 − I4 | Direct-to-BS | 198,218 | 1821 | 2001 | 2076 | 99.32% | 0 |
| v8 − I5 | Energy-gated CH | 258,228 | 2171 | 2641 | 2746 | 99.05% | 92,706 |
| Relative threshold only | All of I1–I5 | 201,565 | 1730 | 2086 | 2166 | 98.16% | 0 |

WHY these results happened: at the center BS, direct-to-BS (I4) is the largest single contributor — without it FND falls from 2621 to 1821 — because in a 100×100 m field with the sink at the center many members are closer to the BS than to their CH, so routing them through a CH only adds the CH's RX + aggregation cost. The energy gate (I5) is second (FND 2171 without it): weak nodes no longer take the heavy CH role, so deaths are postponed and synchronised. Proactive handover (I2) mainly lifts PDR (+0.24 pp) by preventing the packet loss that happens when a CH dies mid-aggregation. I1 and I3 had no measurable effect in the full combination for this seed (they act as safety nets; I1 alone removes all 998 zero-CH rounds from v5b). At the FAR BS, direct-to-BS almost never fires (76 packets), so v8's advantage shrinks: it keeps the best FND (1471 vs 1266 for v7) and PDR (98.66% vs 98.14%) of any far-BS variant, but the CH-to-CH chain of v6/v7 still gives the longer HND/LND (1891/1976 vs 1646/1716). The honest conclusion is that v8 is the best configuration for a center/near sink, while a far sink still benefits from relay chaining.

Practical recommendation: rather than declaring one "final" version, the confirmed results support a context-aware choice — the protocol has two optional components (the CH-to-CH chain and the backup CH) that should be enabled based on deployment geometry and priorities:

| Source | FND | HND | LND | PDR |
| --- | --- | --- | --- | --- |
| Sink center/near, prioritize lifetime → V3 | — | — | 2703 | 97.72% |
| Sink center/near, prioritize reliability → V3+Backup | — | — | 2201 | 98.19% |
| Sink far, prioritize lifetime → V6 Far-BS | — | — | 1711 | 96.81% |
| Sink far, prioritize max reliability → V7 | — | — | 1691 | 97.54% |

9. Updated Practical Recommendations (Including Controlled Variants)

| Priority | Best Variant | Why |
| --- | --- | --- |
| Sink center/near, prioritize lifetime | v3 | Highest LND (2703) with zero added overhead |
| Sink center/near, prioritize reliability | v3 + Backup (v5, interval=5 variant / v7_center) | Best PDR improvement at low lifetime cost |
| Sink far, prioritize lifetime | v6 Far-BS (chain) | Chain gives the largest lifetime gain specifically when BS is far |
| Sink far, prioritize max reliability | v7 / v7_farBS_v2 | Highest PDR among far-BS variants; chain-repair protects against relay breakage |
| Longer duty cycle acceptable, simplicity valued | v7_center_noChain_int10 | No chain complexity; backup alone keeps PDR ~98% even at interval=10 |
| Want shortest routes without a single point of failure | v7_center_multihop | Highest PDR of all variants tested (98.49%); tree structure has no single gateway bottleneck |
| Sink center/near, prioritize lifetime AND reliability | v8 (Center) | Best FND (2621), HND (2711) and PDR (99.22%) of all variants; +33% delivered packets for the same energy |
| Sink far, prioritize first-node lifetime and PDR | v8 (Far BS) | Best far-BS FND (1471) and PDR (98.66%); v7 chain still wins on HND/LND at a far sink |

# 10. Key Findings

Across all three protocol families studied, six findings stand out as the most consequential for anyone designing or extending a clustering protocol for WSNs:

1. CH-selection quality matters more than parameter tuning. Replacing HEED's iterative negotiation with a single-pass energy×connectivity score, recomputed once every five rounds instead of every round, was the single most consequential change in the entire study: the v2→v3 transition alone produced a 94% increase in LND (1393→2703 rounds), dwarfing every other individual modification tested.

2. The cluster-reuse interval is a separate, sensitive lever. Stretching the setup interval from 5 to 15 rounds in isolation (v3→v4) regressed every metric — FND fell from 1730 to 1513, LND from 2703 to 2521, and PDR from 97.72% to 94.67% — because a longer interval leaves the network unprotected for longer whenever a cluster head dies before the next re-election.

3. CH-to-CH relay chaining improves both reach and reliability. Introducing a relay chain (v6) under identical field and radio conditions raised PDR at the field-center base-station placement (97.05%→97.494%) and, independently, gave the far-base-station placement its best FND of the family (1266 rounds) by shortening the average hop distance each cluster head has to cover.

4. Backup failover and path repair make recovery reliable, not just possible. Adding backup CH failover together with chain (and later routing) repair on top of v6 was validated across 100 simulation runs with 16 out of 16 successful backup promotions and chain repairs at the center placement and 21 out of 21 at the far-BS placement, and lifted PDR further at both placements (Center 97.494%→98.432%, Far-BS 97.075%→98.135%).

5. No single version wins on every metric. FND, HND, LND and PDR trade off differently across the family: v7 (Center) is the best-balanced configuration overall, while replacing the ordered relay chain with greedy multihop routing (v7.1) sacrifices a small amount of FND (1551→1491) to reach the family's peak PDR of 98.49% — confirming that the “best” version depends on which metric a deployment actually prioritizes.

6. Preventing CH death beats repairing it, and not every packet needs a CH. The optimized v8 adds proactive CH handover (the CH hands over before it can die), direct-to-BS delivery for nodes nearer the sink than their CH, and an energy gate on CH election on top of v5b's backup failover. At the center BS this raised FND from 1730 to 2621 (+51%), HND from 2091 to 2711 (+30%) and PDR from 97.31% to 99.22%, delivering 33% more packets with the same 50 J — the best result of the entire family. The ablation shows the gain comes mainly from direct-to-BS and the energy gate, which makes it geometry-dependent: at a far sink the relay chain remains the better lifetime choice.

# 11. Limitations & Future Work

Limitations

• The entire study is analytical and simulation-based (NS-3, 100 nodes); no physical hardware or software-defined-radio testbed was used to confirm the results under real-world interference and clock drift.

• Only two base-station placements (field center and a far corner) and a limited set of network sizes were tested; behaviour at substantially larger or smaller deployments was not directly measured.

• Setup-interval sensitivity was demonstrated at two interval lengths (5 and 15 rounds for the core family, 5 and 10 for the multihop variants) but was not exhaustively swept against node count or radio range, so the exact optimal interval for a given deployment remains an open question.

• The backup-failover and chain/routing-repair mechanisms were validated only for single-cluster-head-death events; simultaneous multi-failure scenarios and adversarial/malicious failure patterns were not tested.

• v8's largest gain (direct-to-BS) depends on the sink being inside or near the field; at the far-BS placement it almost never activates and v8's HND/LND fall below the chain-based v7. v8 was evaluated at two BS placements and 8 random topologies only.

Future Work

• Validate the best-performing configurations (v7 and v7.1) on physical WSN hardware or an SDR-based testbed to confirm the simulated gains survive real radio conditions.

• Extend the scalability analysis to 500-1000+ node deployments and irregular (non-uniform) topologies, beyond the 100-node regular field used throughout this study.

• Develop an adaptive setup-interval selection mechanism, and a globally-optimized (rather than greedy) multihop routing scheme, so the interval-length and routing trade-offs identified here can be tuned automatically per deployment.

• Co-evaluate this thesis's hybrid-cryptography security layer together with the best clustering/routing configuration (v7 or v7.1) under adversarial conditions, to confirm that the reliability gains documented here hold up when the network is also under active attack.

• Combine v8's proactive handover, energy gate and direct-to-BS rule with the v7 chain / v7.1 multihop relay so that a single configuration keeps v8's center-BS gains while recovering v7's far-BS lifetime.

# 12. Conclusion

This work set out to design an energy-efficient, secure, and reliable clustering scheme for wireless sensor networks, starting from the well-known LEACH- and HEED-style baselines and evolving an original protocol family through explicit versions v1 through v7.2 and a final optimized v8. The guiding method throughout was to isolate exactly one mechanism change per version — CH-selection formula, cluster-reuse interval, backup failover, relay chaining, or multihop routing — so that each change's individual contribution to FND, HND, LND and PDR could be measured and attributed rather than obscured inside a bundle of simultaneous modifications.

The main conclusion that emerges is that architectural changes — how a cluster head is chosen and how it relays data — mattered far more to the final results than any single parameter tweak: the switch to a single-pass energy×connectivity CH-selection score alone produced a 94% lifetime gain, while stretching a reuse interval in isolation actively made every metric worse. Among the relay-based versions, v7 (Center) is the best-balanced configuration (FND 1551, HND 1976, LND 2066, PDR 98.432%) and v7.1 (Multihop) reaches PDR 98.49%. The final optimized v8 — which prevents CH death through proactive handover, repairs it through backup failover, and lets nodes near the sink bypass their CH — surpasses all of them at the center BS (FND 2621, HND 2711, LND 2761, PDR 99.22%), while the relay chain remains preferable for a far sink. Combined with this thesis's hybrid cryptography layer, these results demonstrate that energy-aware clustering and secure communication can be co-designed without a significant reliability penalty, giving a practical foundation for deploying secure, long-lived wireless sensor networks.